| Co1-At2 | Name: K Muni Kualdeep |
| --- | --- |
| Course code: ITA0329 | Reg No: 192311096 |

Question 1:

Answer:

Number of full-duplex channels = Total bandwidth / (2 × bandwidth per simplex channel)

= 45 MHz / (2 × 25 kHz)

= 45,000 kHz / 50 kHz

= 900

Question 2:

Answer:

Channels per cell = Total channels / Cluster size

= 560 / 7

= 80 channels per cell.

Question 3:

Answer:

Total simultaneous voice channels = Number of carriers × Time slots per carrier

= 100 × 8

= 800 simultaneous voice channels.

Question 4:

Answer:

Power ratio (dB) = 10 log₁₀(Base station power / Handset power)

= 10 log₁₀(80 / 4)

= 10 log₁₀(20)

= 13.01 dB.

Question 5:

Answer:

C/I (dB) = Carrier power (dBm) − Interference power (dBm)

= −60 − (−92)

= 32 dB.

Question 6;

Answer:

Total channels = 42 MHz / 300 kHz = 42,000 / 300 = 140 channels.

Channels per cell = 140 / 7 = 20 channels per cell.

Question 7:

Answer:

Received power varies inversely with distance raised to the path-loss exponent.

P₂ = P₁ × (d₁ / d₂)ⁿ

= 64 × (1 / 2)⁴

= 64 / 16

= 4 W.

Question 8:

Answer:

Transmit power factor = (New radius / Original radius)²

= (2R / R)²

= 2²

= 4.

Question 9:

Answer:

Total channels in cluster = Cluster size × Channels per cell

= 4 × 15

= 60 channels.

Question 10:

Answer:

Convert speed: v = 100 × 1000 / 3600 = 27.78 m/s.

Maximum Doppler shift: fᵈ = (v / c) × fᶜ

= (27.78 / (3 × 10⁸)) × 900 × 10⁶

≈ 83.33 Hz.

Question 11:

Answer:

Time-slot duration = Frame duration / Number of slots

= 5 ms / 8

= 0.625 ms.

Question 12:

Answer:

Number of carrier frequencies = Total bandwidth / Carrier spacing

= 24 MHz / 200 kHz

= 24,000 kHz / 200 kHz

= 120 carrier frequencies.

Question 13:

Answer:

Convert transmit power to dBm:

Pₜ = 10 log₁₀(8 × 1000) = 39.03 dBm.

Received power = Transmit power − Path loss

= 39.03 − 33

= 6.03 dBm.

Question 14:

Answer:

Using frequency in MHz and distance in km:

FSPL = 32.44 + 20 log₁₀(f) + 20 log₁₀(d)

= 32.44 + 20 log₁₀(1900) + 20 log₁₀(2.5)

= 105.98 dB.

Question 15:

Answer:

Total capacity = Number of cells × Calls per cell

= 6 × 250

= 1,500 simultaneous calls.

Question 16:

Answer:

Average call duration = 3 × 60 = 180 seconds.

Using N = Call arrival rate × Average call duration:

N = 4000 × 180 = 720,000 active calls.

Question 17:

Answer:

Total bandwidth = Number of channels × Bandwidth per channel

= 150 × 25 kHz

= 3,750 kHz

= 3.75 MHz.

Question 18:

Answer:

For 30 adjacent bands, the number of gaps is 30 − 1 = 29.

Total guard-band bandwidth = 29 × 40 kHz

= 1,160 kHz

= 1.16 MHz.

Question 19:

Answer:

Cell diameter = 2 × 1.5 = 3 km.

Handoffs per hour ≈ Distance travelled per hour / Cell diameter

= 90 km / 3 km

≈ 30 handoffs per hour.

Question 20:

Answer:

Total base stations = 8 × 40 = 320.

Total user capacity = 320 × 250 = 80,000 users.

Question 21:

Answer:

Convert radius: 400 m = 0.4 km.

Approximate cell area = R² = 0.4² = 0.16 km².

Number of cells = 400 / 0.16 = 2,500 cells.

Question 22:

Answer:

Convert total bandwidth: 6 MHz = 6,000 kHz.

Number of channels = 6,000 / 30 = 200 channels.

Question 23:

Answer:

Traffic per subscriber = (Calls per hour × Average duration in minutes) / 60

= (4 × 2.5) / 60 = 0.1667 Erlangs.

Total offered traffic = 1,200 × 0.1667 ≈ 200 Erlangs.

Question 24:

Answer:

Number of simultaneous users = Total bandwidth / Bandwidth per user

= 40 MHz / 400 kHz

= 40,000 kHz / 400 kHz

= 100 users.

Question 25:

Answer:

Total user capacity = Number of base stations × Users per base station

= 1,200 × 600

= 720,000 users.